`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 08.08.2025 01:39:38

// Design Name:

// Module Name: TLC

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

`define true 1'b1

`define false 1'b0

`define y2rdelay 3

`define r2gdelay 2

module TLC(

output [1:0] hwy,

output [1:0] cntry,

input clock,

input clear,

input x

);

reg [1:0] hwy,cntry;

parameter red = 2'd0,

yellow = 2'd1,

green = 2'd2;

parameter s0 = 3'd0,

s1 = 3'd1,

s2 = 3'd2,

s3 = 3'd3,

s4 = 3'd4;

reg [2:0] state;

reg [2:0] next_state;

always @ (posedge clock)

if(clear)

state <= s0;

else

state <= next_state;

always @(state)

begin

hwy = green;

cntry = red;

case(state)

s0:     ;

s1: hwy = yellow;

s2: hwy = red;

s3: begin

hwy = red;

cntry = green;

end

s4: begin

hwy = red;

cntry = yellow;

end

endcase

end

always @(state or x)

begin

case (state)

s0: if(x)

next_state = s1;

else

next_state = s0;

s1: begin

repeat(`y2rdelay) next_state = s1;

next_state = s2;

end

s2: begin

repeat(`r2gdelay) next_state = s2;

next_state = s3;

end

s3: if(x)

next_state = s3;

else

next_state = s4;

s4: begin

repeat(`y2rdelay) next_state = s4;

next_state = s0;

end

default: next_state =s0;

endcase

end

endmodule